# Report on Network Cards

## Author:Dakata

Report on Network Cards

   Wireless Network Cards
       Wireless NICs are equipped for wireless Ethernet communications and have a fixed or detachable radio antenna.
       They come in four forms:
           CardBus (32-bit PC Card) or ExpressCard (PCIe) cards for laptops without integrated wireless support.
           Mini PCI or PCIe Mini cards that provide wireless, wired Ethernet, and dial-up modem support for laptops.
           PCI or PCIe cards for desktop computers with PCI or PCIe slots.
           USB adapters for both desktop and laptop computers.
       Most laptops with onboard Wi-Fi hardware use the Mini PCI or PCIe Mini interface for the wireless adapter and place the antenna inside the display housing.

   NICs (Network Interface Cards)
       NICs are defined as network interface cards.
       Speed:
           Your NIC should run at the maximum speed your network supports.
           Most Gigabit Ethernet and Fast Ethernet cards support slower speeds (e.g., a 1,000Mbps Gigabit Ethernet card also supports 100Mbps Fast Ethernet or 10Mbps standard Ethernet).
           Look for network cards identified as 10/100 or 10/100/1000 Ethernet for multispeed operation.
           Modern Fast or Gigabit NICs should support full-duplex operation.
               Half-duplex: The network card can only send or only receive data in a single operation.
               Full-duplex: The network card can both receive and send simultaneously, boosting network speed when switches are used. For example, 1,000Mbps Gigabit Ethernet cards in full-duplex mode can achieve a maximum true throughput of 2,000Mbps.
       Bus Types:
           For Gigabit Ethernet (10/100/1000Mbps) networks, adequate performance can be achieved with PCI/PCIe, CardBus/ExpressCard (for laptops), and USB 3.0.
           Integrated NICs on motherboards are typically PCI or PCIe devices.
           USB 2.0 (480Mbps) is not fast enough to fully support 1,000Mbps Gigabit Ethernet, but 100Mbps Ethernet connections work fine on USB 2.0.
           USB 3.0 (5Gbps) is more than fast enough for Gigabit Ethernet adapters.
       Connectors:
           Wired Ethernet adapters commonly use an eight-position, eight-conductor (8P8C) connector, informally known as an RJ-45 connector.
           Fast Ethernet and Gigabit Ethernet twisted-pair cables use these connectors.
           Older adapters might have a single BNC connector (for Thinnet coaxial cables) or a D-shaped 15-pin connector.
       Other details:
           NICs can have onboard BIOS.
           Installation and testing of NICs are mentioned.