# Report on SRAM and DRAM

## Author:Rali123

Report on SRAM and DRAM

SRAM
   SRAM stands for static RAM.
   It is named "static" because it does not require periodic refresh rates like DRAM.
   SRAM is significantly faster than most types of DRAM.
   It is capable of keeping pace with modern processors due to its design.
   SRAM memory can have access times of 0.25ns or less, allowing it to keep pace with processors running 4GHz or faster.
   SRAM design uses a cluster of six transistors for each bit of storage.
   The absence of capacitors in its design means refresh rates are not necessary as there are no charges to lose over time.
   SRAM retains its stored data as long as power is supplied.
   Compared to DRAM, SRAM is faster but has lower density and is more expensive.
   Lower density means SRAM chips are physically larger and store fewer bits.
   The high number of transistors and clustered design make SRAM chips physically larger and more costly to produce than DRAM.
   For example, a high-density DRAM chip might store up to 4Gb (512MB) of RAM, while a similar-sized SRAM chip can only store up to 72Mb (9MB).
   Due to its high cost and physical constraints, SRAM is not typically used as the main memory for PC systems.
   PC designers utilize a small amount of high-speed SRAM memory as cache memory to significantly improve PC performance.
   SRAM is used in Level 1, Level 2, and Level 3 caches.

DRAM
   DRAM chips are produced with densities up to 4Gb (512MB) per chip, requiring at least 4 billion transistors for one transistor per bit.
   The transistor count in memory chips is much higher than in processors due to the consistent arrangement of transistors and capacitors in a grid of simple repetitive structures.
   Each DRAM bit cell's transistor reads the charge state of an adjacent capacitor. A charged capacitor indicates a 1, while no charge indicates a 0.
   The charge in the tiny capacitors constantly drains, necessitating constant refreshing of the memory.
   A momentary power interruption or interference with refresh cycles can cause a DRAM memory cell to lose its charge and data.
   Data loss in a running system can lead to blue screens, global protection faults, corrupted files, and system crashes.
   DRAM is used in PC systems because it is inexpensive and chips can be densely packed, allowing a lot of memory capacity in a small space.
   DRAM is relatively slow, typically much slower than the processor.
   Various DRAM architectures have been developed to improve performance.
   Examples of DRAM architectures include DDR SDRAM, GDDR2 SDRAM, GDDR3 SDRAM, GDDR4 SDRAM, GDDR5 SDRAM, and Rambus DRAM (RDRAM).
   Rambus inline memory modules (RIMMs) are associated with Rambus DRAM.
   DRAM is a type of volatile storage.