# Report on SRAM

## Author:Rali123

Report on SRAM

   SRAM stands for static RAM.
   It is named "static" because it does not need the periodic refresh rates like DRAM.
   SRAM is significantly faster than most types of DRAM.
   It is capable of keeping pace with modern processors due to its design.
   SRAM memory can have access times of 0.25ns or less, allowing it to keep pace with processors running 4GHz or faster.
   SRAM design uses a cluster of six transistors for each bit of storage.
   The absence of capacitors in its design means refresh rates are not necessary because there are no capacitors to lose their charges over time.
   As long as there is power, SRAM remembers what is stored.
   Compared to DRAM, SRAM is much faster but also much lower in density and much more expensive.
   The lower density means that SRAM chips are physically larger and store fewer bits overall.
   The high number of transistors and the clustered design mean that SRAM chips are both physically larger and much more expensive to produce than DRAM chips.
   For example, a high-density DRAM chip might store up to 4Gb (512MB) of RAM, whereas similar-sized SRAM chips can only store up to 72Mb (9MB).
   The high cost and physical constraints have prevented SRAM from being used as the main memory for PC systems.
   PC designers have found a way to use SRAM to dramatically improve PC performance.
   They design in a small amount of high-speed SRAM memory, used as cache memory.
   SRAM is used in Level 1 cache, Level 2 cache, and Level 3 cache.