# Pentium MMX and SDRAM Report

## Author:Rali123

Pentium MMX and SDRAM Report

Pentium MMX

   MMX instructions are designed to process highly parallel and repetitive sequences, frequently found in multimedia operations, more efficiently.
   MMX originally worked only on integer values.
   It used the floating-point unit for processing, which led to time loss when transitioning to floating-point operations.
   These initial drawbacks of MMX were later addressed and corrected in subsequent additions by Intel and AMD.
   Intel licensed its MMX capabilities to other companies, including AMD and Cyrix (which was later acquired by VIA), allowing them to integrate MMX technology into their own Intel-compatible processors.
   The Pentium II processor is essentially a Pentium Pro enhanced with MMX technology instructions and a modified cache design.
   The Pentium II incorporated 57 new multimedia-related instructions that originated from MMX processors.
   An MMX upgrade also involved doubling the internal L1 cache, increasing it from 16KB total in the Pentium Pro to 32KB total in the Pentium II.
   Streaming SIMD Extensions (SSE), introduced with the Pentium III processor, served as an update to MMX and was sometimes referred to as MMX-2 before its official release.
   Unlike MMX, SSE instructions enable floating-point calculations and utilize a dedicated unit within the processor, rather than sharing the standard floating-point unit.

SDRAM

   SDRAM, which stands for synchronous DRAM, is a JEDEC standard memory type that operates in synchronization with the memory bus.
   It delivers information through high-speed bursts using a high-speed clocked interface.
   SDRAM reduces latency compared to asynchronous DRAM because its signals are synchronized with the motherboard clock.
   Motherboard chipset support is essential for SDRAM to be usable in systems.
   Intel chipsets, starting with the 430VX and 430TX in 1996, began to support industry-standard SDRAM.
   In 1998, with the introduction of the 440BX chipset, SDRAM became more popular than EDO memory.
   Variations of SDRAM include DDR SDRAM, DDR2 SDRAM, DDR3 SDRAM, and DDR4 SDRAM.