# Pentium MMX, SDRAM, ROM Memory, and Network Cards Report

## Author:Rali123

Pentium MMX, SDRAM, ROM Memory, and Network Cards Report

Pentium MMX

   MMX instructions are designed to process highly parallel and repetitive sequences, frequently found in multimedia operations, more efficiently.
   MMX originally worked only on integer values.
   It used the floating-point unit for processing, which led to time loss when transitioning to floating-point operations.
   These initial drawbacks of MMX were later addressed and corrected in subsequent additions by Intel and AMD.
   Intel licensed its MMX capabilities to other companies, including AMD and Cyrix (which was later acquired by VIA), allowing them to integrate MMX technology into their own Intel-compatible processors.
   The Pentium II processor is essentially a Pentium Pro enhanced with MMX technology instructions and a modified cache design.
   The Pentium II incorporated 57 new multimedia-related instructions that originated from MMX processors.
   An MMX upgrade also involved doubling the internal L1 cache, increasing it from 16KB total in the Pentium Pro to 32KB total in the Pentium II.
   Streaming SIMD Extensions (SSE), introduced with the Pentium III processor, served as an update to MMX and was sometimes referred to as MMX-2 before its official release.
   Unlike MMX, SSE instructions enable floating-point calculations and utilize a dedicated unit within the processor, rather than sharing the standard floating-point unit.

SDRAM

   SDRAM, which stands for synchronous DRAM, is a JEDEC standard memory type that operates in synchronization with the memory bus.
   It delivers information through high-speed bursts using a high-speed clocked interface.
   SDRAM reduces latency compared to asynchronous DRAM because its signals are synchronized with the motherboard clock.
   Motherboard chipset support is essential for SDRAM to be usable in systems.
   Intel chipsets, starting with the 430VX and 430TX in 1996, began to support industry-standard SDRAM.
   In 1998, with the introduction of the 440BX chipset, SDRAM became more popular than EDO memory.
   Variations of SDRAM include DDR SDRAM, DDR2 SDRAM, DDR3 SDRAM, and DDR4 SDRAM.

ROM Memory

   ROM stands for Read-Only Memory.
   The PC begins executing BIOS instructions 16 bytes from the end of the ROM, a deliberate design in Intel's x86 processors.
   A JMP (jump) instruction at this address directs the processor to the actual start of the ROM, typically around F0000h (approximately 64KB earlier in the memory map).
   This design allows ROMs to be of various sizes while consistently residing at the higher end of addresses within the first megabyte of the memory address space.
   During the Power On Self Test (POST) phase of the boot process, the motherboard ROM automatically scans and reads adapter card ROMs.
   The motherboard ROM searches a specific area of RAM (addresses C0000–DFFFFh) for adapter ROMs, looking for '55AAh' signature bytes to identify their beginning.
   All adapter ROMs must start with '55AAh' to be recognized by the motherboard.
   The third byte within an adapter ROM indicates its size in 512-byte units, known as paragraphs.
   The fourth byte marks the actual beginning of the driver programs.
   For testing purposes, the motherboard ROM sums all bytes in the ROM and divides the total by the number of bytes; the expected remainder is 100h.
   Programmers typically use a "fill" byte at the end of an adapter ROM to ensure the checksum calculation is correct.

Network Cards

   Network Interface Cards (NICs) are used for network connections.
   They can support various speeds, with modern Gigabit Ethernet and Fast Ethernet cards often compatible with multiple speeds (e.g., 10/100 or 10/100/1000 Mbps).
   NICs operate in either half-duplex (sending or receiving data at one time) or full-duplex (simultaneous sending and receiving) modes; full-duplex can increase network throughput, especially when used with switches.
   Common bus types for network adapters include PCI/PCIe (for integrated and expansion cards), CardBus/ExpressCard (for laptops), and USB adapters.
   USB 2.0 provides insufficient bandwidth for Gigabit Ethernet but is suitable for 100Mbps Ethernet connections, whereas USB 3.0 (5Gbps) is fast enough for Gigabit Ethernet.
   Wired Ethernet adapters typically feature an 8P8C (RJ-45) connector.
   Wireless network cards (wireless NICs) are available in forms such as CardBus/ExpressCard, Mini PCI/PCIe Mini cards (often integrated into laptops), PCI/PCIe cards for desktops, and USB adapters.
   Wireless NICs are equipped with either a fixed or detachable radio antenna.
   Laptops with integrated Wi-Fi commonly use Mini PCI or PCIe Mini interfaces for their wireless adapters, with the antenna usually placed within the display housing.