# SRAM and Cache Memory Report

## Author:Dakata

SRAM and Cache Memory Report

SRAM (Static RAM)
- Compared to DRAM
- Explained
- Hits/misses
- Used in Level 1 cache
- Used in Level 2 cache
- Used in Level 3 cache

Cache Memory
- Holds data the processor is most likely to need in advance.
- Enables the processor to continue working at full or near-full speed without waiting for data from slower main memory.
- Usually made up of static RAM (SRAM) integrated into the processor die.
- Older systems used cache chips installed on the motherboard.
- Recent low-cost processor designs typically include two levels of processor/memory cache: Level 1 (L1) and Level 2 (L2).
- Mid-range and high-end designs also have Level 3 cache.
- All modern processors starting with the 486 family include an integrated L1 cache and controller.
- The integrated L1 cache size varies from 8KB for the original 486DX up to 128KB or more in the latest processors.
- Multicore processors include separate L1 caches for each processor core.
- L1 cache is divided into equal amounts for instructions and data.
- L1 cache is always built into the processor die and runs at the full-core speed of the processor internally.
- Cache memory can be accessed with no wait states.
- When a request is made for data from a specific main memory address, the address is compared against address tag entries in the cache tag RAM.
    - If the requested main memory address is found in the tag (a "hit"), the corresponding location in the cache is returned.
    - If the requested address is not found in the address tag entries (a "miss"), the data must be retrieved from main memory.
- Cache can be mapped in various ways:
    - Fully associative mapped cache: Address is compared against all address tag entries. Offers the highest hit ratio but is the slowest.
    - Direct-mapped cache: Specific main memory addresses are preassigned to specific line locations in the cache. Faster (only one tag needs to be checked) but can result in more misses.
    - Set associative cache: A modified direct-mapped cache that is a compromise between a fully associative cache and a direct-mapped cache. An `n`-way set associative cache has `n` sets.
- The contents of the cache must always be in sync with the contents of main memory.
- The internal cache in the 486 family was a "write-through" cache, meaning information written to the cache was automatically written through to main memory.
- Pentium and later chips have an internal "write-back" cache, where both reads and writes are cached.
- Improved cache designs are "nonblocking".