# Report on Pentium D and SDRAM

## Author:Gerginov

Report on Pentium D and SDRAM

Pentium D
   Introduced in 2005, code-named Smithfield.
   Based on the Pentium 4 Prescott core, utilizing two Prescott cores for a dual-core design.
   Cores communicate via the MCH (North Bridge) chip on the motherboard.
   Requires specific chipsets such as Intel’s 945, 955X, and 975X series, E7230 workstation chipset, or NVIDIA’s nForce 4 series. It is incompatible with Intel 915 and 925 chipsets.
   Key features include:
       Clock speeds ranging from 2.66GHz to 3.6GHz.
       533MHz or 800MHz processor bus.
       EM64T 64-bit extensions.
       Execute Disable Bit support.
       Manufactured using a 65- or 90-nanometer process.
       2MB/4MB L2 cache (1MB/2MB per core).
       Uses Socket T (LGA775).
   Models 830, 840, and 9xx include Enhanced Intel Speed Step Technology for cooler and quieter operation.
   The Pentium Extreme Edition is a high-end variant with HT Technology and unlocked clock multipliers, but lacks Enhanced Intel Speed Step Technology.
   It was Intel's first-generation dual-core processor for desktop PCs.
   The subsequent Core 2 processor family, with its Core microarchitecture, offered 40% better performance and was 40% more energy efficient than the Pentium D.

SDRAM
   SDRAM stands for synchronous DRAM.
   It is a JEDEC standard type of DRAM that operates in synchronization with the memory bus.
   It delivers information in high-speed bursts through a high-speed clocked interface.
   SDRAM reduces latency compared to asynchronous DRAM because its signals are synchronized with the motherboard clock.
   Motherboard chipset support is necessary for its use in systems.
   Intel chipsets, starting with the 430VX and 430TX in 1996, began supporting industry-standard SDRAM.
   By 1998, with the release of the 440BX chipset, SDRAM became the most popular memory type, surpassing EDO.
   Its faster architecture became the standard for system bus speeds of 100MHz and above, replacing EDO RAM which was limited to CPU bus speeds up to 66MHz.