# Report on Pentium D

## Author:Gerginov

Report on Pentium D

   Introduced in 2005, code-named Smithfield.
   Based on the Pentium 4 Prescott core, utilizing two Prescott cores for a dual-core design.
   Cores communicate via the MCH (North Bridge) chip on the motherboard.
   Requires specific chipsets such as Intel’s 945, 955X, and 975X series, E7230 workstation chipset, or NVIDIA’s nForce 4 series. It is incompatible with Intel 915 and 925 chipsets.
   Key features include:
       Clock speeds ranging from 2.66GHz to 3.6GHz.
       533MHz or 800MHz processor bus.
       EM64T 64-bit extensions.
       Execute Disable Bit support.
       Manufactured using a 65- or 90-nanometer process.
       2MB/4MB L2 cache (1MB/2MB per core).
       Uses Socket T (LGA775).
   Models 830, 840, and 9xx include Enhanced Intel Speed Step Technology for cooler and quieter operation.
   The Pentium Extreme Edition is a high-end variant with HT Technology and unlocked clock multipliers, but lacks Enhanced Intel Speed Step Technology.
   It was Intel's first-generation dual-core processor for desktop PCs.
   The subsequent Core 2 processor family, with its Core microarchitecture, offered 40% better performance and was 40% more energy efficient than the Pentium D.