# SRAM and ROM Report

## Author:Rali123

SRAM and ROM Report

   SRAM (static RAM) does not require periodic refresh rates like DRAM.
   SRAM is significantly faster than most types of DRAM, with access times of 0.25ns or less, capable of keeping pace with processors running 4GHz or faster.
   The design of SRAM uses a cluster of six transistors for each bit of storage, meaning refresh rates are not necessary as there are no capacitors to lose charges.
   As long as there is power, SRAM remembers what is stored.
   Compared to DRAM, SRAM is much lower in density and much more expensive.
   SRAM chips are physically larger and store fewer bits (e.g., up to 72Mb or 9MB compared to a high-density DRAM chip storing up to 4Gb or 512MB).
   Due to high cost and physical constraints, SRAM is not used as the main memory for PC systems.
   SRAM is used as high-speed cache memory to improve PC performance.

   ROM stands for read-only memory.
   The PC memory map includes ROM BIOS.
   Intel's x86 processors are designed such that the PC starts executing BIOS instructions 16 bytes from the end of the ROM.
   A JMP (jump) instruction at this address directs the processor to the actual beginning of the ROM, typically around F0000h (about 64KB earlier in the memory map).
   This design allows ROMs to be of any size while residing at the upper end of addresses in the first megabyte of the memory address space.
   Adapter card ROMs are automatically scanned and read by the motherboard ROM during the Power On Self Test (POST).
   The motherboard ROM scans a designated area of RAM (addresses C0000–DFFFFh) for adapter ROMs, identifying them by 55AAh signature bytes that mark the start of a ROM.
   All adapter ROMs must begin with 55AAh to be recognized by the motherboard.
   The third byte in a ROM indicates its size in 512-byte units, known as paragraphs.
   The fourth byte signifies the actual start of the driver programs.
   The motherboard ROM uses the size byte for testing, performing a checksum where the sum of all bytes in the ROM divided by the number of bytes should yield a remainder of 100h. Programmers typically use a "fill" byte to achieve the correct checksum.