# Difference between Core 2 duo and Nehalem

## Author:Dakata

Report: Difference between Core 2 Duo and Nehalem

   Core 2 Duo
       Part of the Core 2 family.
       Standard dual-core processors.
       Dual-core versions have 291 million transistors.
       Include 1MB or 2MB of L2 cache per core.
       Initially built on a 65nm process, with 45nm versions released later.
       Highlights of the Core microarchitecture include:
           Wide dynamic execution (33% wider internal execution cores).
           Intelligent power capability (advanced power-gating).
           Advanced smart cache (multicore optimized L2 cache).
           Smart memory access (memory disambiguation).
           Advanced digital media boost (improves SSE instruction execution).

   Nehalem (Core i Processors)
       The first generation of Core i processors, which replaced the Core 2 family.
       Introduced with the Core i7 in November 2008.
       Key features include the integration of the memory controller into the processor.
       Some models integrate the entire North Bridge, including an optional graphics processor, into the processor die.
       Initially built on a 45nm process, with later versions built on an improved 32nm process.
       All Nehalem processors support DDR3 memory and include L3 cache.
       Some models support HT Technology.
       Two main variants:
           High-end versions use Socket LGA1366, include a triple-channel DDR3 memory controller, and use QPI (Quick Path Interconnect) to connect to the North Bridge.
           Mainstream models use Socket LGA1156, include a fully integrated North Bridge (dual-channel DDR3 memory controller, graphics interface, optional full-blown graphics processor), and use a 2GBps DMI as the FSB connection to the South Bridge.