# Difference between DRAM and SRAM

## Author:Gerginov

Report on the Difference Between DRAM and SRAM

*   **DRAM (Dynamic RAM)**
    *   Requires constant refreshing due to charges in tiny capacitors draining.
    *   Inexpensive to produce.
    *   High density, allowing a lot of memory capacity in a small space (e.g., up to 4Gb per chip).
    *   Relatively slow, typically much slower than the processor.
    *   Used in PC systems as main memory.
    *   Memory cells consist of a transistor and an adjacent capacitor.
    *   A momentary power interruption or interference with refresh cycles can cause data loss.

*   **SRAM (Static RAM)**
    *   Does not need periodic refresh rates.
    *   Much faster than DRAM, capable of keeping pace with modern processors (access times of 0.25ns or less).
    *   Lower density, meaning SRAM chips are physically larger and store fewer bits (e.g., up to 72Mb for a similar-sized chip as a 4Gb DRAM).
    *   Much more expensive to produce than DRAM.
    *   Uses a cluster of six transistors for each bit of storage, without capacitors.
    *   Remembers what is stored as long as there is power.
    *   Impractical for use as main system memory due to cost and physical constraints, but used as high-speed cache memory.