# Network cards

## Author:Dakata

Network cards

   Your Network Interface Card (NIC) should operate at the maximum speed you desire for your network.
   Most Gigabit Ethernet and Fast Ethernet cards can also support slower speeds, meaning a 1,000Mbps (Gigabit Ethernet) card can also operate at 100Mbps (Fast Ethernet) or 10Mbps (standard Ethernet).
   To verify multispeed operation, look for network cards labeled 10/100 or 10/100/1000 Ethernet.
   Modern Fast or Gigabit NICs support full-duplex operation.
       Half-duplex allows the network card to either send or receive data in a single operation.
       Full-duplex allows the network card to receive and send data simultaneously.
       Full-duplex operations can boost network speed, for example, 1,000Mbps Gigabit Ethernet cards running in full-duplex mode can achieve a maximum true throughput of 2,000Mbps.
   For Gigabit Ethernet (10/100/1000Mbps) networks, suitable bus types include:
       PCI/PCIe: Integrated NICs on most motherboards use these.
       CardBus/ExpressCard: Used in laptop computers.
   These bus types support Gigabit Ethernet adapters without limiting throughput.
   USB 2.0 (480Mbps) is not fast enough to fully support Gigabit Ethernet’s 1,000Mbps bandwidth, but it can support 100Mbps Ethernet connections.
   USB 3.0 (5Gbps) is fast enough to support Gigabit Ethernet adapters.
   Wired Ethernet adapters typically feature an 8P8C (RJ-45) connector, resembling a large telephone jack, used for Fast Ethernet and Gigabit Ethernet twisted-pair cables.
   Older adapters might have a single BNC connector (for Thinnet coaxial cables) or a D-shaped 15-pin connector.
   Wireless network cards (NICs) have a fixed or detachable radio antenna.
   Wireless NICs are available in four forms:
       CardBus (32-bit PC Card) or ExpressCard (PCIe) cards for laptops without integrated wireless support.
       Mini PCI or PCIe Mini cards for laptops, providing wireless, wired Ethernet, and dial-up modem support.
       PCI or PCIe cards for desktop computers.
       USB adapters for both desktop and laptop computers.
   Most laptop computers with onboard Wi-Fi use the Mini PCI or PCIe Mini interface for the wireless adapter, with the antenna placed inside the display housing.
   An access point translates conventional Ethernet signals into wireless Ethernet signals, broadcasting them to wireless NICs, and performs the reverse for signals from wireless NICs to the conventional Ethernet network.
   Access points are not required for direct peer-to-peer networking (ad hoc mode) but are necessary for a shared internet connection or a connection with another network (infrastructure mode).