# Power supply basics

## Author:Rali123

## Power Supply Basics

   The power supply is a critical and often overlooked component in a PC, considered the foundation of the system.
   It supplies electrical power to every other component in the system and is one of the most failure-prone components.
   A malfunctioning power supply can cause other components to malfunction or even damage them by delivering improper or erratic voltages.
   Primary Function and Operation:
       Converts electrical power from the wall socket (120V 60Hz AC or 240V 50Hz AC) to power the computer circuitry can use (+3.3V, +5V, and +12V DC).
       Some power supplies require manual switching between input ranges, while others auto-switch.
   Constant Voltage Switching Power Supply Unit (PSU):
       Constant voltage: The power supply outputs the same voltage to internal components regardless of the AC input voltage or power supply wattage.
       Switching: Refers to the design and power regulation technique that provides an efficient, inexpensive power source, generates minimal heat, and maintains a small size and low price.
   Voltage Rails:
       The PSU typically supplies +3.3V, +5V, and +12V to the system, referred to as rails.
       Multiple wires carry these specific voltages, tied to a single rail (or tap) in the PSU, to manage the load more efficiently and cheaply than a single thick wire.
   Input Voltage Caution:
       If a power supply does not auto-switch input voltages, ensure the setting is correct.
       Plugging into a 120V outlet with the supply set to 240V will not cause damage but will prevent proper operation.
       Plugging into a 240V outlet with the supply set to 120V can cause damage.
       North American power companies supply split-phase 240V AC (two 120V legs), and resistive voltage drops can lead to actual voltages of 220V/110V at outlets.
   High-Quality Power Supplies:
       More likely to withstand power disturbances like brownouts.
       Can protect systems from damage during a 100% power outage, brownout, or a spike of up to 2,500V applied to the AC input.
       Have extremely low current leakage to ground (less than 500 microamps), which is a safety feature for outlets with missing or improperly wired ground lines.
   Other Power Supply Specifications:
       Mean Time Between Failures (MTBF) or Mean Time To Failure (MTTF): The calculated average interval, in hours, that the power supply is expected to operate before failure.