# Report on Pentium 4

## Author:KrumchoTati

Report on Pentium 4

   General Features and Architecture
       Software compatible with previous Intel 32-bit processors.
       Some versions support EM64T (64-bit extensions) and Execute Disable Bit (buffer overflow protection).
       Processor (front-side) bus runs at 400MHz, 533MHz, 800MHz, or 1,066MHz.
       Arithmetic logic units (ALUs) run at twice the processor core frequency.
       Features hyper-pipelined (20-stage or 31-stage) technology.
       Supports HT Technology in all 2.4GHz and faster processors with an 800MHz bus, and all 3.06GHz and faster processors with a 533MHz bus. HT Technology allows a single processor to run two threads simultaneously.
       Deep out-of-order instruction execution and enhanced branch prediction.
       Introduces the "NetBurst microarchitecture," a marketing term describing hyper-pipelined technology, a rapid execution engine, a high-speed system bus, and an execution trace cache.
       Hyper-pipelined technology doubles or triples instruction pipeline depth compared to Pentium III, enabling higher clock speeds.
       Rapid execution engine allows ALUs to run at twice the processor core frequency, enabling instructions to execute in half a clock cycle.
       High-speed processor bus is a quad-pumped bus (100MHz, 133MHz, 200MHz, or 266MHz system clock, transferring data four times per cycle), resulting in effective rates of 400MHz, 533MHz, 800MHz, or 1,066MHz.
       The 64-bit wide bus provides throughput rates of 3,200MBps, 4,266MBps, 6,400MBps, or 8,533MBps.
       Pipelined internal architecture (20-stage or 31-stage) breaks instructions into more substages, similar to a RISC processor.

   Cache
       8KB or 16KB L1 cache plus 12K micro-op execution trace cache.
       256KB, 512KB, 1MB, or 2MB of on-die, full-core speed 256-bit-wide L2 cache with eight-way associativity.
       L2 cache can handle all physical memory and supports ECC.
       2MB of on-die, full-speed L3 cache (specific to Extreme Edition).
       Execution trace cache is a high-performance Level 1 cache that stores approximately 12K decoded micro-operations, increasing performance by removing the instruction decoder from the main execution pipeline.

   Instruction Sets
       SSE2-SSE plus 144 new instructions for graphics and sound processing (Willamette and Northwood cores).
       SSE3-SSE2 plus 13 new instructions for graphics and sound processing (Prescott core).
       Enhanced floating-point unit.

   Sockets and Compatibility
       Initially used Socket 423 (423 pins, 39x39 SPGA).
       Later versions used Socket 478.
       Final versions use Socket T (LGA775), which added support for features like EM64T, Execute Disable Bit, and Intel Virtualization Technology.
       Celeron D versions are available for Socket 478 and Socket T (LGA775), offering lower-cost systems compatible with the Pentium 4.

   Pentium 4 Extreme Edition
       Introduced in November 2003, it was the first desktop PC processor to incorporate L3 cache.
       Essentially a revamped version of the Prestonia core Xeon workstation/server processor.
       Features 512KB of L2 cache and 2MB of L3 cache, increasing transistor count to 178 million.
       Expensive to produce due to large die based on 130-nanometer process.
       Targeted toward the gaming market due to its performance benefits for power-hungry 3D games.
       Revised versions in 2004 were based on the 90-nanometer (0.09-micron) Pentium 4 Prescott core, with a larger 2MB L2 cache (without L3 cache in these versions).
       Produced in both Socket 478 and Socket T form factors.