# Report on SRAM Cache and DRAM

## Author:KrumchoTati

Report on SRAM Cache and DRAM

   SRAM Cache
       Runs at speeds close to or equal to the processor.
       Processor usually directly reads from and writes to it.
       Acts as a high-speed buffer between the processor and slower DRAM.
       Cache controller anticipates processor memory needs and preloads data.
       Data can be retrieved from SRAM cache rather than slower main memory (DRAM).
       Effectiveness is measured by a "hit ratio" (data found in cache).
       A "cache miss" occurs when data is not in the cache, requiring retrieval from slower main memory.
       SRAM stands for static RAM; it does not need periodic refresh rates like DRAM.
       SRAM is much faster than DRAM and more capable of keeping pace with modern processors.
       SRAM memory can have access times of 0.25ns or less, keeping pace with processors running 4GHz or faster.
       Designed with a cluster of six transistors per bit of storage, meaning no capacitors and thus no refresh rates are needed.
       SRAM remembers what is stored as long as there is power.
       It is much faster than DRAM but also much lower in density and more expensive.
       SRAM chips are physically larger and store fewer bits than DRAM.
       Historically, SRAM cache began to enter PC system designs with 386 processors (1986-1987) as processors exceeded 16MHz.
       Modern systems often use Level 1 (L1), Level 2 (L2), and Level 3 (L3) caches.
       L1 cache is "integral" or "internal," built directly into the processor die and runs at full processor core speed.
       All 486 and higher processors incorporate integral L1 cache.
       L2 cache was originally "external," installed on the motherboard in 386, 486, and first-generation Pentium systems.

   DRAM (Dynamic RAM)
       Lower-speed main memory.
       Data in high-speed cache memory is resupplied from DRAM.
       Up until the late 1990s, DRAM was limited to about 60ns (16MHz) in speed.
       When processors exceeded 16MHz, DRAM could no longer keep pace, necessitating SRAM cache.
       When a processor reads from main memory (DRAM), it slows down by executing "wait states."
       Each DRAM bit cell uses a transistor to read the charge state of an adjacent capacitor.
       If the capacitor is charged, it's a 1; no charge indicates a 0.
       Capacitor charges constantly drain, requiring constant refreshing.
       Momentary power interruption or interference with refresh cycles can cause data loss.
       Data loss in DRAM can lead to blue screens, global protection faults, corrupted files, and system crashes.
       DRAM is used in PC systems because it is inexpensive and chips can be densely packed, offering high memory capacity in a small space.
       It is relatively slow compared to processors.
       DRAM chips can store up to 4Gb (512MB) per chip.

I cannot provide specific information about "Pentium D" because the required information is not present in the database.