# Report on SRAM Cache, DRAM, SDRAM, and Cache Memory

## Author:KrumchoTati

Report on SRAM Cache, DRAM, SDRAM, and Cache Memory

   SRAM (Static RAM) and Cache Memory
       SRAM cache operates at speeds close to or equal to the processor.
       The processor usually directly reads from and writes to SRAM cache.
       During read operations, data in the high-speed cache memory is resupplied from the lower-speed main memory (DRAM) in advance.
       SRAM is a high-speed buffer made up of SRAM that directly feeds the processor.
       It acts as a buffer between the processor and the slower DRAM in the system.
       The cache controller anticipates processor memory needs and preloads the high-speed cache memory with data.
       When the processor requests a memory address, data can be retrieved from the high-speed cache rather than slower main memory.
       Cache effectiveness is expressed by a hit ratio (ratio of cache hits to total memory accesses).
       A "hit" occurs when the needed data is preloaded into the cache.
       A "cache miss" occurs when the cache controller did not anticipate the need, and the data must be retrieved from slower main memory.
       Retrieving data from main memory causes the processor to wait longer, executing "wait states."
       SRAM is named "static" because it does not require periodic refresh rates like DRAM.
       SRAM is significantly faster than most types of DRAM and can keep pace with modern processors.
       SRAM memory can have access times of 0.25ns or less, supporting processors at 4GHz or faster.
       SRAM design uses a cluster of six transistors per bit of storage, eliminating the need for capacitors and thus refresh rates.
       As long as there is power, SRAM retains its stored data.
       SRAM is faster but much lower in density and more expensive than DRAM.
       Early PC systems (1986-1987, 386 processors at 16-20MHz) were among the first to use cache memory as processors exceeded DRAM's speed limits.
       Modern systems typically use two or three stages of cache: Level 1 (L1), Level 2 (L2), and Level 3 (L3).
       L1 cache is "integral" or "internal," built directly into the processor die, runs at full processor core speed, and is the fastest cache.
       All 486 and higher processors incorporate integral L1 cache.
       L2 cache was originally "external," installed on the motherboard in 386, 486, and first-generation Pentium systems, running at motherboard and CPU bus speeds.

   DRAM (Dynamic RAM)
       DRAM is a lower-speed main memory.
       Up until the late 1990s, DRAM speed was limited to about 60ns (16MHz).
       When processors surpassed 16MHz, DRAM could no longer keep pace, leading to the introduction of SRAM cache.
       If a processor running at 3.6GHz on a 1,333MHz bus reads from main memory, it effectively slows down to 1,333MHz.
       The slowdown is managed by the processor executing "wait states."

   SDRAM
       Related memory modules mentioned include:
           168-pin SDRAM DIMMs
           184-pin DDR DIMMs
           240-pin DDR2 DIMMs
           240-pin DDR3 DIMMs

I cannot provide a report on "Pentium D" because the required information is not present in the database.