# 486 Processors

## Author:Gerginov

486 Processors
   The Intel 80486 (normally abbreviated as 486) was a major leap forward in speed.
   The additional power available in the 486 fueled tremendous growth in the software industry.
   The 486 finally made the graphical user interface (GUI) of Windows and OS/2 a realistic option for daily computer users.
   The 486 is a family of processors, consisting of DX, SX, and other variations.
   Four main features make 486 processors roughly twice as fast as an equivalent MHz 386 chip:
       Reduced instruction-execution time: A single instruction in the 486 takes an average of only two clock cycles to complete, compared to an average of more than four cycles on the 386.
       Internal (Level 1) cache: The built-in cache has a hit ratio of 90–95%, describing how often zero-wait-state read operations occur.
       Burst-mode memory cycles: Up to 16 bytes of contiguous, sequential memory data can be transferred in as little as five cycles instead of eight cycles or more.
       Built-in (synchronous) enhanced math coprocessor (some versions): The math coprocessor runs synchronously with the main processor and executes math instructions in fewer cycles than previous designs did. In DX-series chips, this provides two to three times greater math performance than an external 387 math coprocessor chip.
   The 486 chip is about twice as fast as the 386 at the same clock rate, which led to the rapid decline of the 386 in the marketplace.