# Pentium IV Report

## Author:Rali123

Pentium IV Report

   Software compatible with previous Intel 32-bit processors.
   Some versions supporting EM64T (64-bit extensions) and Execute Disable Bit (buffer overflow protection).
   Processor (front-side) bus that runs at 400MHz, 533MHz, 800MHz, or 1,066MHz.
   Arithmetic logic units (ALUs) that run at twice the processor core frequency.
   Hyper-pipelined (20-stage or 31-stage) technology.
   HT Technology support in all 2.4GHz and faster processors running an 800MHz bus and all 3.06GHz and faster processors running a 533MHz bus.
   Deep out-of-order instruction execution.
   Enhanced branch prediction.
   8KB or 16KB L1 cache plus 12K micro-op execution trace cache.
   256KB, 512KB, 1MB, or 2MB of on-die, full-core speed 256-bit-wide L2 cache with eight-way associativity.
   L2 cache that can handle all physical memory and supports ECC.
   2MB of on-die, full-speed L3 cache (Extreme Edition).
   SSE2-SSE plus 144 new instructions for graphics and sound processing (Willamette and Northwood).
   SSE3-SSE2 plus 13 new instructions for graphics and sound processing (Prescott).
   Enhanced floating-point unit.
   Multiple low-power states.
   Intel abandoned Roman numerals for a standard Arabic numeral 4 designation to identify the Pentium 4.
   Introduces a new architecture called NetBurst microarchitecture, which describes hyper-pipelined technology, a rapid execution engine, a high-speed system bus, and an execution trace cache.
   Hyper-pipelined technology doubles or triples the instruction pipeline depth compared to the Pentium III.
   Rapid execution engine enables the two integer ALUs to run at twice the processor core frequency.
   The 400MHz/533MHz/800MHz/1,066MHz system bus is a quad-pumped bus, transferring data four times per clock cycle.
   Execution trace cache is a high-performance Level 1 cache that stores approximately 12K decoded micro-operations.
   High-speed processor bus is a 100MHz, 133MHz, 200MHz, or 266MHz quad-pumped bus (4x) for a 400MHz, 533MHz, 800MHz, or 1,066MHz effective rate.
   Bus is 64 bits (8 bytes) wide, resulting in throughput rates of 3,200MBps, 4,266MBps, 6,400MBps, or 8,533MBps.
   Internal architecture has a 20-stage or 31-stage pipeline, breaking down individual instructions into many more substages.
   HT Technology enables a single processor to run two threads simultaneously, acting as if it were two processors.
   Initially used Socket 423 (423 pins in a 39x39 SPGA arrangement).
   Later versions used Socket 478.
   Final versions use Socket T (LGA775), with additional pins for EM64T, Execute Disable Bit, and Intel Virtualization Technology.
   Celeron and Celeron D versions are available for Socket 478 and Socket T (LGA775).
   Voltage selection is made via an automatic voltage regulator module installed on the motherboard.

Pentium 4 Extreme Edition:
   Introduced in November 2003, notable for being the first desktop PC processor to incorporate L3 cache.
   Basically a revamped version of the Prestonia core Xeon workstation/server processor.
   Has 512KB of L2 cache and 2MB of L3 cache, increasing transistor count to 178 million.
   Based on the 130-nanometer process, making it expensive to produce.
   Targeted toward the gaming market.
   Revised versions introduced in 2004 are based on the 90-nanometer (0.09-micron) Pentium 4 Prescott core but with a larger 2MB L2 cache (instead of 512KB).
   Pentium 4 Extreme Edition processors based on the Prescott core do not have L3 cache.
   Produced in both Socket 478 and Socket T form factors.