# Report on Pentium II and Pentium III

## Author:Rali123

Report on Pentium II and Pentium III

Pentium III
   The Pentium III processor was released in February 1999.
   It introduced several new features to the P6 family, essentially having the same core as a Pentium II with the addition of SSE instructions and integrated on-die L2 cache in later versions.
   SSE consists of 70 new instructions enhancing performance for advanced imaging, 3D, streaming audio, video, and speech-recognition applications.
   The processor was originally based on Intel’s 0.25-micron CMOS process technology, starting with more than 9.5 million transistors.
   In late 1999, Intel shifted to a 0.18-micron process die (code-named Coppermine) and added 256KB of on-die L2 cache, increasing the transistor count to 28.1 million.
   The last version of the Pentium III (code-named Tualatin) used a 0.13-micron process and had 44 million transistors.
   Motherboards made before the Tualatin-core versions generally do not support this processor due to logical pinout changes.
   The Pentium III was manufactured in speeds from 450MHz through 1.4GHz.
   Server versions with larger or faster cache were known as the Pentium Xeon.
   It incorporates a 32KB L1 cache.
   It features either half-core speed 512KB L2 cache or full-core speed on-die 256KB or 512KB L2 with cacheability for up to 4GB of addressable memory space.
   The PIII can be used in dual-processing systems with up to 64GB of physical memory.
   It includes a self-reportable processor serial number for security, authentication, and system management applications. This feature can be disabled in the system BIOS due to privacy concerns.
   Pentium III processors were initially available in Intel’s SECC2 form factor, which replaced the older SECC packaging.
   The SECC2 package covers only one side of the chip, allowing for better heatsink attachment and less overall weight.
   Intel later switched to the FC-PGA package, which is less expensive and allows for more direct heatsink attachment for better cooling.
   The FC-PGA version plugs into Socket 370 but can be used in Slot 1 with a slotkey adapter.
   All Pentium III processors have either 512KB or 256KB of L2 cache, running at either half-core or full-core speed.
   Pentium III Xeon versions have 512KB, 1MB, or 2MB of L2 cache that runs at full-core speed.
   Pentium III Xeon is a more expensive version designed for servers and workstations.
   All PIII processor L2 caches can cache up to 4GB of addressable memory space and include ECC capability.

Pentium II
   Intel revealed the Pentium II in May 1997.
   Prior to its official unveiling, it was popularly referred to by its code name, Klamath.
   The Pentium II is essentially the same sixth-generation processor as the Pentium Pro, with MMX technology added.
   MMX technology included double the L1 cache and 57 new MMX instructions.
   From a physical standpoint, the Pentium II used a departure from previous processors with its SEC cartridge (SECC) design.
   The processor, along with several L2 cache chips, is mounted on a small circuit board (like an oversized-memory SIMM).
   The circuit board is sealed in a metal and plastic cartridge.
   The cartridge is plugged into the motherboard through an edge connector called Slot 1, resembling an adapter card slot.
   Two variations of these cartridges are SECC (single edge contact cartridge) and SECC2.
   The SECC2 version was cheaper to make with fewer parts and allowed for a more direct heatsink attachment for better cooling.
   Intel transitioned from SECC to SECC2 in early 1999; later PII chips and subsequent Slot 1 PIII chips used the SECC2 design.
   Using separate chips on a circuit board allowed Intel to build the Pentium II less expansively than the Pentium Pro.
   Intel could also use cache chips from other manufacturers and more easily vary cache amounts.
   The Pentium II has the same multiprocessor scalability as the Pentium Pro and an integrated L2 cache.
   It includes 57 new multimedia-related instructions from MMX processors and improved capability for repetitive loop commands.
   The internal L1 cache was doubled from the Pentium Pro (from 16KB total to 32KB total in the Pentium II).