# Report on Pentium II

## Author:Rali123

Report on Pentium II

   Intel revealed the Pentium II in May 1997.
   Prior to its official unveiling, the Pentium II processor was popularly referred to by its code name, Klamath.
   The Pentium II is essentially the same sixth-generation processor as the Pentium Pro, with MMX technology added.
   MMX technology included double the L1 cache and 57 new MMX instructions.
   The Pentium II chip is characterized by its SEC cartridge (SECC) design.
   The processor, along with several L2 cache chips, is mounted on a small circuit board, which is then sealed in a metal and plastic cartridge.
   The cartridge is plugged into the motherboard through an edge connector called Slot 1.
   Variations on these cartridges are called SECC (single edge contact cartridge) and SECC2.
   The SECC2 version was cheaper to make and allowed for a more direct heatsink attachment to the processor for better cooling.
   Intel transitioned from SECC to SECC2 in the beginning of 1999.
   By using separate chips mounted on a circuit board, Intel could build the Pentium II much less expensively than the Pentium Pro.
   This design also allowed Intel to use cache chips from other manufacturers and more easily vary the amount of cache.
   The Pentium II has the same multiprocessor scalability as the Pentium Pro, as well as integrated L2 cache.
   It includes the 57 new multimedia-related instructions carried over from the MMX processors and the capability to process repetitive loop commands more efficiently.
   The internal L1 cache was doubled from the Pentium Pro (from 16KB total to 32KB total in the Pentium II).