# Report on Pentium III

## Author:Rali123

Report on Pentium III

   The Pentium III processor was released in February 1999.
   It introduced several new features to the P6 family, essentially having the same core as a Pentium II with the addition of SSE instructions and integrated on-die L2 cache in later versions.
   SSE consists of 70 new instructions enhancing performance for advanced imaging, 3D, streaming audio, video, and speech-recognition applications.
   The processor was originally based on Intel’s 0.25-micron CMOS process technology, starting with more than 9.5 million transistors.
   In late 1999, Intel shifted to a 0.18-micron process die (code-named Coppermine) and added 256KB of on-die L2 cache, increasing the transistor count to 28.1 million.
   The last version of the Pentium III (code-named Tualatin) used a 0.13-micron process and had 44 million transistors.
   Motherboards made before the Tualatin-core versions generally do not support this processor due to logical pinout changes.
   The Pentium III was manufactured in speeds from 450MHz through 1.4GHz.
   Server versions with larger or faster cache were known as the Pentium Xeon.
   It incorporates a 32KB L1 cache.
   It features either half-core speed 512KB L2 cache or full-core speed on-die 256KB or 512KB L2 with cacheability for up to 4GB of addressable memory space.
   The PIII can be used in dual-processing systems with up to 64GB of physical memory.
   It includes a self-reportable processor serial number for security, authentication, and system management applications. This feature can be disabled in the system BIOS due to privacy concerns.
   Pentium III processors were initially available in Intel’s SECC2 form factor, which replaced the older SECC packaging.
   The SECC2 package covers only one side of the chip, allowing for better heatsink attachment and less overall weight.
   Intel later switched to the FC-PGA package, which is less expensive and allows for more direct heatsink attachment for better cooling.
   The FC-PGA version plugs into Socket 370 but can be used in Slot 1 with a slotkey adapter.
   All Pentium III processors have either 512KB or 256KB of L2 cache, running at either half-core or full-core speed.
   Pentium III Xeon versions have 512KB, 1MB, or 2MB of L2 cache that runs at full-core speed.
   Pentium III Xeon is a more expensive version designed for servers and workstations.
   All PIII processor L2 caches can cache up to 4GB of addressable memory space and include ECC capability.