# Report on Pentium Processors

## Author:Rali123

Report on Pentium Processors

   The Pentium III processor was released in February 1999 and introduced new features to the P6 family.
   The Pentium III is essentially the same core as a Pentium II with the addition of SSE instructions and integrated on-die L2 cache in later versions.
   SSE consists of 70 new instructions that enhance the performance and possibilities of advanced imaging, 3D, streaming audio, video, and speech-recognition applications.
   Originally based on Intel’s 0.25-micron CMOS process technology, the PIII core started with over 9.5 million transistors.
   In late 1999, Intel shifted to a 0.18-micron process die (code-named Coppermine) and added 256KB of on-die L2 cache, increasing the transistor count to 28.1 million.
   The last version of the Pentium III (code-named Tualatin) uses a 0.13-micron process and has 44 million transistors.
   The Pentium III was manufactured in speeds from 450MHz through 1.4GHz, and also in server versions with larger or faster cache known as the Pentium Xeon.
   The Pentium III incorporates a 32KB L1 cache and either half-core speed 512KB L2 cache or full-core speed on-die 256KB or 512KB L2 with cacheability for up to 4GB of addressable memory space.
   The PIII can be used in dual-processing systems with up to 64GB of physical memory.
   A self-reportable processor serial number provides security, authentication, and system management applications with a tool for identifying individual systems, which can be disabled in the system BIOS due to privacy concerns.
   Pentium III processors were made available in Intel’s SECC2 form factor, which replaced the older SECC packaging, allowing for better heatsink attachment and less overall weight.
   The Pentium is fully compatible with previous Intel processors but features twin data pipelines, enabling it to execute two instructions at the same time.
   Intel calls this capability "superscalar technology," which provides additional performance compared to the 486.
   With superscalar technology, the Pentium can execute many instructions at a rate of two instructions per cycle and is one of the first CISC chips to be considered superscalar.
   The two instruction pipelines within the chip are called the u-pipe and v-pipe. The u-pipe is the primary pipe and can execute all integer and floating-point instructions. The v-pipe is a secondary pipe that can execute only simple integer instructions and certain floating-point instructions.
   The process of operating on two instructions simultaneously in the different pipes is called pairing.
   The Pentium processor has a branch target buffer (BTB), which employs a technique called branch prediction, minimizing stalls in one or more of the pipes.
   Branch prediction attempts to predict whether a program branch will be taken and then fetches the appropriate instructions, enabling the Pentium to keep both pipelines operating at full speed.