# Include more information about dual core architecture in the file

## Author:Dakata

Report on Pentium D

   Pentium D processors were introduced in 2005.
   They used the code name Smithfield before their introduction.
   Pentium D processors are based on the Pentium 4 Prescott core.
   To bring dual-core processors to market as quickly as possible, Intel used two Prescott cores in each Pentium D or Pentium Extreme Edition processor.
   Each core communicates with the other via the MCH (North Bridge) chip on the motherboard.
   Intel 915 and 925 chipsets and some third-party chipsets made for the Pentium 4 cannot be used with Pentium D or Pentium Extreme Edition.
   Intel's 945 series, 955X, and 975X desktop chipsets, and the E7230 workstation chipset, are the first Intel chipsets to support these processors.
   The nForce 4 series from NVIDIA also works with these processors.
   Major features of the Pentium D include:
       Clock speeds of 2.66GHz–3.6GHz
       533MHz or 800MHz processor bus
       EM64T 64-bit extensions
       Execute Disable Bit support
       65- or 90-nanometer manufacturing process
       2MB/4MB L2 cache (1MB/2MB per core)
       Socket T (LGA775)
   The 830, 840, and 9xx models also include Enhanced Intel Speed Step Technology.
   Enhanced Intel Speed Step Technology provides a wide range of processor speeds in response to workload and thermal issues, resulting in cooler and quieter PC operation.
   The Pentium Extreme Edition is a high-end version of the Pentium D with differences such as:
       Support for HT Technology, enabling each core to simulate two processor cores for better performance with multithreaded applications.
       No support for Enhanced Intel Speed Step Technology.
       Includes unlocked clock multipliers for easy overclocking.
   The Pentium D processor was Intel's first-generation dual-core processor for desktop PCs.
   The Core 2 Duo processor provides 40% better performance and is 40% more energy efficient than the previous generation Pentium D processor.