# Pentium Extreme Edition Report

## Author:Rali123

Pentium Extreme Edition Report

   Pentium 4 Extreme Edition (Pentium 4EE)
       Introduction: Introduced in November 2003.
       L3 Cache: Notable as the first desktop PC processor to incorporate L3 cache (2MB).
       L2 Cache: Features 512KB of L2 cache.
       Transistor Count: Increases transistor count to 178 million transistors, making the die significantly larger than the standard Pentium 4.
       Origin: Basically a revamped version of the Prestonia core Xeon workstation/server processor, which had used L3 cache since November 2002.
       Cost: Expensive to produce due to the large die based on the 130-nanometer process, reflected in its high selling price.
       Target Market: Targeted toward the gaming market, where the additional cache helps power-hungry 3D games more than standard business applications.
       Revised Versions (2004): Introduced based on the 90-nanometer (0.09-micron) Pentium 4 Prescott core, but with a larger 2MB L2 cache in place of the 512KB L2 cache design used by the standard Prescott-core Pentium 4. These revised processors do not have L3 cache.
       Form Factors: Produced in both Socket 478 and Socket T form factors.

   Pentium Extreme Edition (High-End Version of Pentium D)
       Nature: A high-end version of the Pentium D processor.
       HT Technology: Supports HT Technology, allowing each core to simulate two processor cores for enhanced performance with multithreaded applications.
       Intel Speed Step Technology: Enhanced Intel Speed Step Technology is not supported.
       Overclocking: Includes unlocked clock multipliers, which facilitates easy overclocking.