# Pentium II Report (Revised)

## Author:Gerginov

Pentium II Report (Revised)
- Intel revealed the Pentium II in May 1997.
- Prior to its official unveiling, it was referred to by its code name, Klamath.
- The Pentium II is essentially the same sixth-generation processor as the Pentium Pro, with MMX technology added.
- MMX Technology:
    - It included double the L1 cache (from 16KB total in the Pentium Pro to 32KB total in the Pentium II).
    - It added 57 new MMX instructions.
    - These 57 new multimedia-related instructions, carried over from the MMX processors, enhance the capability to process repetitive loop commands more efficiently.
- From a physical standpoint, the Pentium II chip is characterized by its SEC cartridge (SECC) design.
- The processor, along with several L2 cache chips, is mounted on a small circuit board, which is then sealed in a metal and plastic cartridge.
- The cartridge is plugged into the motherboard through an edge connector called Slot 1.
- Two variations of these cartridges are SECC and SECC2.
- The SECC2 version was cheaper to make, used fewer parts, and allowed for a more direct heatsink attachment for better cooling.
- Intel transitioned from SECC to SECC2 in early 1999.
- Later PII chips and Slot 1 PIII chips use the improved SECC2 design.
- Using separate chips mounted on a circuit board allowed Intel to build the Pentium II much less expensively than the Pentium Pro.
- This design also allowed Intel to use cache chips from other manufacturers and more easily vary the amount of cache in future processors.
- The Pentium II has the same multiprocessor scalability as the Pentium Pro, as well as integrated L2 cache.