# Network adapters

## Author:Gerginov

Report on Network Adapters

   Ethernet Ports and Speed:
       Network Interface Cards (NICs) should operate at the maximum desired network speed.
       Most Gigabit Ethernet cards (1,000Mbps) support slower speeds like Fast Ethernet (100Mbps) and standard Ethernet (10Mbps).
       Multispeed operation is indicated by "10/100" or "10/100/1000 Ethernet."
       Modern Fast or Gigabit NICs support full-duplex operation.
           Half-duplex: Card can only send or only receive data at a time.
           Full-duplex: Card can both receive and send simultaneously, boosting network speed when switches are used. For instance, 1,000Mbps Gigabit Ethernet in full-duplex can achieve 2,000Mbps throughput.

   Bus Type for Gigabit Ethernet Adapters:
       For Gigabit Ethernet (10/100/1000Mbps) networks, adequate performance is provided by:
           PCI/PCIe (integrated NICs in most motherboards)
           CardBus/ExpressCard (for laptop computers)
       These buses support Gigabit Ethernet adapters without limiting throughput.
       Integrated network adapters use PCI or PCI Express buses.
       USB 2.0 (480Mbps) is not fast enough for full Gigabit Ethernet (1,000Mbps) support, but it works for 100Mbps Ethernet.
       USB 3.0 (5Gbps) is fast enough for Gigabit Ethernet adapters, and such adapters are available from vendors.

   Wired Network Adapter Connectors:
       Typically, wired Ethernet adapters have an eight-position, eight-conductor (8P8C) connector, informally known as an RJ-45, resembling a large telephone jack.
       Fast Ethernet and Gigabit Ethernet twisted-pair cables use RJ-45 connectors.
       Older adapters might have a single BNC connector (for Thinnet coaxial cables) or a D-shaped 15-pin connector.

   Wireless Network Cards (NICs):
       An access point is a device with 8P8C (RJ-45) ports that can connect to a 10BASE-T or 10/100/1000 Ethernet network. It contains a radio transceiver, encryption, and communications software to translate Ethernet signals into wireless signals and vice-versa.
       Access points are often integrated into wireless routers, which may also include a switch, cable/DSL modem, and other features.
       Multiple access points can be used with an Ethernet switch for wide area coverage, allowing users to roam without losing network contact. Some access points can create a wireless backbone by communicating directly via radio waves.
       Wireless Ethernet range extenders can boost weak Wi-Fi signals.
       Access points are required for a shared Internet connection or connection to another network (infrastructure mode). They are not necessary for direct peer-to-peer networking (ad hoc mode).
       Wireless clients in ad hoc mode cannot connect to the Internet unless a station is connected to a bridge or uses another network adapter as a bridge.
       Wireless NICs have fixed or detachable radio antennas and come in four forms:
           CardBus (32-bit PC Card) or ExpressCard (PCIe) cards for laptops without integrated wireless.
           Mini PCI or PCIe Mini cards for laptops, providing wireless, wired Ethernet, and dial-up modem support.
           PCI or PCIe cards for desktop computers.
           USB adapters for both desktop and laptop computers.
       Most laptops with onboard Wi-Fi use Mini PCI or PCIe Mini interfaces for the wireless adapter, with the antenna placed inside the display housing.