# Pentium II

## Author:Gerginov

Pentium II Report

   Intel revealed the Pentium II in May 1997.
   Prior to its official unveiling, it was popularly referred to by its code name, Klamath.
   The Pentium II is essentially the same sixth-generation processor as the Pentium Pro, with MMX technology added.
   MMX technology included double the L1 cache (from 16KB total to 32KB total) and 57 new MMX instructions.
   Physically, the Pentium II chip departed from previous processors by using an SEC cartridge (SECC) design.
   The processor, along with several L2 cache chips, is mounted on a small circuit board, which is then sealed in a metal and plastic cartridge.
   The cartridge plugs into the motherboard through an edge connector called Slot 1.
   There are two variations of these cartridges: SECC (single edge contact cartridge) and SECC2.
   The SECC2 version was cheaper to make due to fewer overall parts and allowed for more direct heatsink attachment for better cooling.
   Intel transitioned from SECC to SECC2 in early 1999; all later PII chips and subsequent Slot 1 PIII chips used the SECC2 design.
   Using separate chips mounted on a circuit board allowed Intel to build the Pentium II less expensively than the Pentium Pro's multiple die within a package.
   This design also allowed Intel to use cache chips from other manufacturers and more easily vary the amount of cache in future processors.
   The Pentium II has the same multiprocessor scalability and integrated L2 cache as the Pentium Pro.
   It includes the 57 new multimedia-related instructions from the MMX processors and the capability to process repetitive loop commands more efficiently.