# Pentium IV cores specifications

## Author:Rali123

Pentium IV Cores Specifications Report

   Software compatible with previous Intel 32-bit processors.
   Some versions support EM64T (64-bit extensions) and Execute Disable Bit (buffer overflow protection).
   Processor (front-side) bus runs at 400MHz, 533MHz, 800MHz, or 1,066MHz.
   Arithmetic logic units (ALUs) run at twice the processor core frequency.
   Features Hyper-pipelined (20-stage or 31-stage) technology.
   HT Technology support in all 2.4GHz and faster processors running an 800MHz bus and all 3.06GHz and faster processors running a 533MHz bus.
   Includes deep out-of-order instruction execution.
   Enhanced branch prediction.
   Has 8KB or 16KB L1 cache plus 12K micro-op execution trace cache.
   Offers 256KB, 512KB, 1MB, or 2MB of on-die, full-core speed 256-bit-wide L2 cache with eight-way associativity.
   L2 cache can handle all physical memory and supports ECC.
   2MB of on-die, full-speed L3 cache (Extreme Edition).
   Supports SSE2-SSE plus 144 new instructions for graphics and sound processing (Willamette and Northwood).
   Supports SSE3-SSE2 plus 13 new instructions for graphics and sound processing (Prescott).
   Enhanced floating-point unit.
   Multiple low-power states.
   Utilizes the NetBurst microarchitecture, which includes hyper-pipelined technology, a rapid execution engine, a high-speed system bus, and an execution trace cache.
   The system bus is a quad-pumped bus, with effective rates of 400MHz, 533MHz, 800MHz, or 1,066MHz, resulting in throughput rates from 3,200MBps to 8,533MBps.
   Initially used Socket 423, later versions used Socket 478, and final versions used Socket T (LGA775) supporting features like EM64T, Execute Disable Bit, and Intel Virtualization Technology.
   Pentium 4 Extreme Edition versions include L3 cache (initially 2MB with 512KB L2 cache) or a larger 2MB L2 cache (Prescott-based versions).