# Video adapters

## Author:Rali123

Video Adapters Report

   DAC (RAMDAC)
       Converts RAM-based digital images into signals for analog monitor connections.
       Speed is measured in MHz; faster conversion leads to higher vertical refresh rates.
       Speeds in high-performance video adapters range from 300MHz to 400MHz.
       Most video card chipsets integrate the RAMDAC function into the 3D accelerator chip.
       Dual-display-capable cards may use a separate RAMDAC for different refresh rates on a second display.
       Integrated graphics systems include RAMDAC in the North Bridge or GMCH chip.
       Increased RAMDAC speed allows higher resolutions with flicker-free refresh rates (72Hz–85Hz or above) on CRT displays.
       RAMDAC speeds of 300MHz or above typically display flicker-free (75Hz or above) at resolutions up to 1920×1200 on CRT displays.
       RAMDAC functionality is not used by digital displays, and many GPUs no longer list RAMDAC speeds.

   Video Display Interfaces
       Connect a PC to a display through two main interfaces:
           System interface: Connection between the video adapter and the PC.
           Display interface: Connection between the video adapter and the display.
       Standardized versions of these interfaces ensure compatibility and interchangeability.

   System Interface Standards
       Older standards (now obsolete due to slow performance): IBM MicroChannel Architecture (MCA), Interactive Multimedia Association (ISA), Enhanced Interactive Multimedia Association (EISA), and VESA local bus (VL-Bus).
       Current standards: PCIe interface.
       Still common in older systems: Accelerated Graphics Port (AGP).
       Found in some old systems or AGP-based systems with a second adapter: Peripheral Component Interconnect (PCI)-based adapters.

   Video BIOS
       Video adapters have a separate BIOS from the main system BIOS.
       It's a read-only memory (ROM) chip containing basic instructions for an interface between hardware and software.
       Enables the system to display information during POST and boot sequences before software drivers load.
       Can be upgraded in some cases, especially for high-performance gaming cards.
       Normally uses an electrically erasable programmable read-only memory (EEPROM) chip.
       Many video cards use a surface-mounted BIOS chip.
       Upgradable BIOS using software is called a "flash BIOS."
       Some graphics cards (e.g., AMD Radeon HD 6900 and 7970 GPUs) include two BIOS chips: one non-upgradeable and one user-upgradeable for overclocked performance.
       Video BIOS upgrades are also known as firmware upgrades.

   Video Processor (Video Chipset, Video Graphics Processor, or GPU)
       The core of any video adapter, defining its functions and performance levels.
       Video adapters built with the same chipset have the same basic capabilities.
       Performance can vary between cards with the same chipset due to differences in clock speeds of the chipset, memory, output ports, and the amount/type of memory installed.
       Software drivers for operating systems and applications are primarily written with the chipset in mind.
       A driver for a particular chipset can usually be used on other adapters from the same chipset or chipset families.
       Almost all video adapters on the market today include chipsets with 3D acceleration features.