# Report on SRAM and Pentium 3

## Author:KrumchoTati

Report on SRAM and Pentium 3

Report on SRAM:
   SRAM stands for static RAM and does not require periodic refresh rates like DRAM.
   It is significantly faster than DRAM and can keep pace with modern processors, including those running at 4GHz or faster, with access times of 0.25ns or less.
   SRAM's design uses a cluster of six transistors for each bit of storage and does not include capacitors, eliminating the need for refresh rates.
   As long as power is supplied, SRAM retains its stored data.
   Compared to DRAM, SRAM is much faster but has a lower density and is considerably more expensive.
   SRAM chips are physically larger and store fewer bits; for example, a high-density DRAM chip might hold 4Gb (512MB), whereas a similar-sized SRAM chip might only store up to 72Mb (9MB).
   Due to its high cost and physical constraints, SRAM is not used as the main memory in PC systems.
   Instead, PC designers utilize a small amount of high-speed SRAM as cache memory (e.g., Level 1, Level 2, and Level 3 cache) to significantly enhance PC performance.

Report on Pentium 3:
   The Pentium III processor was released in February 1999, introducing new features to the P6 family, essentially a Pentium II core with the addition of SSE instructions.
   SSE (Streaming SIMD Extensions) comprises 70 new instructions designed to dramatically improve performance in advanced imaging, 3D, streaming audio, video, and speech-recognition applications.
   Initially based on Intel’s 0.25-micron CMOS process technology with over 9.5 million transistors.
   Later versions included the 0.18-micron process die (code-named Coppermine) in late 1999, which added 256KB of on-die L2 cache, increasing the transistor count to 28.1 million.
   The final version, code-named Tualatin, used a 0.13-micron process and featured 44 million transistors; older motherboards might not support it due to logical pinout changes.
   Pentium III processors were manufactured in speeds ranging from 450MHz to 1.4GHz.
   Server versions, known as Pentium Xeon, were available with larger or faster cache.
   It incorporated a 32KB L1 cache and an L2 cache that was either half-core speed 512KB or full-core speed on-die 256KB or 512KB, supporting up to 4GB of addressable memory space.
   The Pentium III could be used in dual-processing systems, supporting up to 64GB of physical memory.
   It featured a self-reportable processor serial number for security and system management, a feature that could be disabled in the system BIOS due to privacy concerns.
   Available in SECC2 and later FC-PGA (Socket 370) packaging; SECC2 allowed for better heatsink attachment, and FC-PGA was less expensive and provided more direct heatsink contact.
   Pentium III Xeon versions specifically offered 512KB, 1MB, or 2MB of full-core speed L2 cache and included ECC capability, with all PIII L2 caches capable of caching up to 4GB of addressable memory space.