# Motherboard Report

## Author:Gerginov

Motherboard Report

   To find motherboard port addresses, consult documentation or Windows Device Manager.
   PCI and PCIe-based devices reduce device conflicts.
   Motherboard Selection Criteria:
       Think like an engineer, considering present and future usage and upgrades.
       Technical support at a professional level is important.
       Check for easy-to-find and up-to-date manual and driver downloads.
       Motherboard chipset: The backbone of a system; compare features for desired functionality (e.g., faster memory, PCIe 2.x or 3.x cards, SATA 6Gbps drives, optional RAID capabilities). Recommended chipsets support the latest Intel or AMD multi-core processors (e.g., Intel 7x or AMD AM3+ or FM2 chipsets).
       Processor socket: Dictates compatible processor makes and models. Choose a motherboard with a socket that supports the desired processor. Main sockets include Socket AM3+, FM1, and FM2 for AMD processors, and Sockets LGA1155 and LGA 2011 for Intel processors. Check motherboard specifications for supported processors.
       Memory: Type and amount depend on the motherboard. Most support DDR3 memory (single-, dual-, triple-, or quad-channel). Number of sockets, supported speeds, and other variables depend on board specifications. DDR4 SDRAM was not expected until 2013-2014.
       Form factor: Indicates size and shape; must be compatible with the chassis/case and power supply. ATX and microATX form factors are recommended for flexibility, performance, reliability, and ease of use.
   The nForce 900 series for AMD included the 980a SLI chipset, supporting processors in Sockets AM2, AM2+, and AM3 (Phenom, Phenom II, Athlon X2, Athlon). It supports PCIe version 2, has 35 PCIe lanes, and can support three-way SLI.
   NVIDIA also supported SLI on AMD’s 900-series chipsets.
   Super I/O Chips:
       Integrates devices formerly found on separate expansion cards.
       Newer motherboards lacking legacy ports have less need for them.
       Features like CMOS RAM are often integrated into the South Bridge chipset component.
       In late-model chipsets, the South Bridge chip includes Super I/O functionality and supports ports like USB, SATA, and IEEE-1394, replacing legacy ports.
   Motherboard Connectors: A modern motherboard has a variety of connectors, some of which (power supply, serial and parallel ports, keyboard/mouse) are covered in other chapters.