# Motherboard and BIOS Report

## Author:Gerginov

Motherboard and BIOS Report

   Most motherboard manufacturers have incorporated the latest UEFI specifications into their motherboards.
   BIOS Hardware/Software:
       Important drivers must be active during boot time.
       Minimum hard disk drivers must be preloaded into ROM on the motherboard or an adapter card for the system to boot.
       The original PC’s motherboard ROM was designed to scan slots for adapter cards with ROMs, which were then executed during startup.
       Cards almost always with onboard ROM/BIOS include:
           Video cards/GPUs.
           RAID cards (for bootable arrays).
           Network cards (with boot ROM or IPL ROM) supporting diskless workstations (Net PCs, NCs, thin clients, smart terminals); Intel developed PXE for network booting.
           ATA/Serial ATA (SATA) upgrade boards (may require onboard BIOS for bootable drives).
   Upgrading the BIOS:
       Motherboard manufacturers tailor BIOS code to specific hardware on each board.
       BIOS resides in ROM chips on the motherboard and contains code specific to the motherboard model or revision.
       BIOS upgrades must come from the motherboard manufacturer or a BIOS upgrade company supporting the specific motherboard.
       Upgrades fix bugs, solve problems, or add support for newer processors, UEFI, larger internal hard drives, bootable optical/USB drives, and faster booting.
       Older systems may require BIOS updates for newer OSs.
       Check the board manufacturer’s website periodically for updates and fixes.
       Check available BIOS upgrades before installing new hardware or software, especially processors and OSs.
   Where to Get Your BIOS Update:
       Most BIOS upgrades are downloaded from the system or motherboard manufacturer’s website.
       BIOS manufacturers (e.g., Phoenix, AMI, Award) do not offer direct upgrades because the BIOS on a motherboard is a custom version licensed and customized by the motherboard manufacturer.
       Upgrades must be obtained from the motherboard or system manufacturer as they are customized for that specific board or system.
   Determining Your BIOS Version:
       To seek a BIOS upgrade, you need the make and model of the motherboard (or system) and the version of the existing BIOS.
       The BIOS information is often displayed briefly during system power-up; pressing the Pause key can freeze the display.
       If not displayed automatically, the BIOS version may be found in the BIOS Setup program.
       Many PCs show a splash screen instead of the typical POST screen; a specific key or keys (specific to the BIOS manufacturer) must be pressed to enter BIOS Setup.
       BIOS splash screens can often be customized (e.g., Intel Integrator Toolkit, GIGABYTE Face-Wizard).