# Nehalem Report

## Author:Dakata

Nehalem Report

   The Nehalem architecture is the basis for the first generation of Intel Core i processors, replacing the Core 2.
   Key features of the Nehalem microarchitecture include:
       Integration of the memory controller into the processor.
       In some models, the entire North Bridge is integrated, including an optional graphics processor in a separate core on the processor die.
   The first Core i Series processor, the Core i7, was introduced in November 2008 and was based on Nehalem.
   Initially built on a 45nm process, later Core i Series processors using Nehalem were built on an improved 32nm process, leading to smaller die, lower power consumption, and greater performance.
   All Nehalem-based processors support DDR3 memory and include L3 cache.
   Some Nehalem models include support for HT Technology.
   There are two main variants in the first-generation (Nehalem) Core i Series Family:
       High-end versions use Socket LGA1366. These include a triple-channel DDR3 memory controller and a high-performance FSB called QPI (Quick Path Interconnect) that connects to the North Bridge component (I/O Hub or IOH) on the motherboard. The IOH implements the PCIe graphics interface.
       More mainstream models use Socket LGA1156. These include a fully integrated North Bridge, comprising a dual-channel DDR3 memory controller, graphics interface, and an optional full-blown graphics processor (Larrabee). Because the entire North Bridge functionality is integrated into the processor, Socket LGA1156 chips use a slower 2GBps DMI as the FSB connection to the South Bridge component on the motherboard.